AC POWER METER

Background Project | UG4 | Electrical Engineering

TEAM MEMBERS Dwaipayan Shikari ¦ Roll No: 002210801231 Shramana Mondal ¦ Roll No: 002210801239 Prattay Barua ¦ Roll No: 002210801246 Jayrup Das ¦ Roll No: 002210801131

# 1. Project Overview

This document details the development of a custom AC Power Meter built as a Background Project for UG4 Electrical Engineering. The project evolved through iterative design — from a basic breadboard prototype to a fully custom PCB — demonstrating practical skills in embedded systems, analog circuit design, PCB layout, and firmware development.

Project Status: Functional but incomplete — current measurement is working correctly; voltage measurement requires a fix due to soldering issues on the RP2040.

## Development Timeline

| Phase | Description |
| --- | --- |
| Version 1 (Prototype) | ESP32-based breadboard build with ZMPT voltage sensor and current transformer, HTTP web server for live data. |
| Testing | Calibration of voltage and current readings; identified noise issues with ESP32 ADC. |
| PCB Design | Redesigned around RP2040 for cleaner ADC; designed schematic and layout in EasyEDA Pro. |
| Manufacturing | PCB fabricated by JLCPCB (China), components hand-soldered locally. |
| Current Build | Functional unit with working current measurement; voltage pin under debugging. |

# 2. Version 1 — Prototype

## Operating Principle

The first version was a straightforward implementation focused on core measurement functionality. The design used two primary sensing elements:

- ZMPT Voltage Sensor — Converts mains AC (230V) to a 0–5V signal centered at 2.5V (representing zero crossing). A 2/3 voltage divider was applied to step this down further to the 3.3V logic level of the ESP32.

- Current Transformer — The load-carrying wire is looped through the transformer core. The induced current is converted to a measurable voltage via a bleed resistor.

## Signal Conditioning — Voltage Divider

A simple resistor divider (R1 / R2) was used to scale the ZMPT output to safe ADC input levels:

Voltage Divider Ratio: 2/3 Input: 0–5V (from ZMPT) Output: 0–3.3V (to ESP32 ADC pin)

## Data Access

An HTTP-based web server was implemented on the ESP32 using its onboard Wi-Fi module. This allowed any device on the local network to view live voltage and current readings through a web browser — a quick, simple, and robust solution for initial prototyping.

# 3. Testing Phase

Testing was conducted using a variable autotransformer to sweep the input voltage from 0V to 220V. Voltage and current readings were compared against a reference meter. Key findings:

- Voltage accuracy was acceptable across the range, with a consistent ~5% under-read.

- Current readings were weaker than expected due to choosing a current transformer with too high a turns ratio, which increased the signal-to-noise ratio unfavourably.

- These errors were addressed in the revised firmware through software calibration coefficients.

## Measurement Data

| Actual (V) | Calculated (V) | Current (A) | Current Calculated (A) |
| --- | --- | --- | --- |
| 220 | 211 | 0.32 | 0.46 |
| 210 | 204 | 0.31 | 0.44 |
| 200 | 192 | 0.3 | 0.4 |
| 190 | 182 | 0.29 | 0.4 |
| 180 | 173 | 0.28 | 0.39 |
| 170 | 163 | 0.27 | 0.37 |
| 160 | 154 | 0.26 | 0.35 |
| 150 | 143 | 0.25 | 0.32 |
| 140 | 133 | 0.24 | 0.3 |
| 130 | 124 | 0.23 | 0.29 |
| 120 | 115 | 0.22 | 0.222 |
| 110 | 105 | 0.21 | 0.21 |
| 100 | 96 | 0.19 | 0.185 |
| 90 | 85 | 0.18 | 0.178 |
| 80 | 77 | 0.17 | 0.157 |
| 70 | 67 | 0.16 | 0.13 |
| 60 | 57 | 0.14 | 0.125 |
| 50 | 47 | 0.13 | 0.12 |
| 40 | 37 | 0.11 | 0.08 |
| 30 | 28 | 0.09 | 0.06 |
| 20 | 18 | 0.07 | 0.047 |
| 10 | 9 | 0.05 | 0.02 |
| 0 | 0 | 0.01 | 0 |

# 4. PCB Design & Redesign

## Motivation

After the initial prototype was proven to work, the team chose to redesign the hardware into a proper PCB for a cleaner, more permanent build. The redesign also addressed a key limitation of the prototype:

Issue: The ESP32's ADC is noisy and non-linear, making accurate RMS calculations difficult. Solution: Replaced with RP2040 (Raspberry Pi), which features a cleaner 12-bit ADC. When used with MicroPython, the scale is mapped to 0–65535 (16-bit equivalent).

## Schematic — Circuit Blocks

- Voltage Measurement: DC 1.65V bias + AC swing + input pin protection diodes

- Current Measurement: DC 1.65V bias + AC swing

- Power Supply: Dual-input — USB and HLK-PM03 AC-DC converter (100–240V input, 3.3V/1A output)

- MCU: Raspberry Pi RP2040 with 2MB external flash (W25Q32BSSIG)

## PCB Fabrication

EDA Tool: EasyEDA Pro

Manufacturer: JLCPCB, China (PCB only)

Assembly: Hand soldered manually — leaded solder and hot air

Connectors: KF128-5.08-2P-AA screw terminals for AC input

# 5. Firmware

## Sampling Strategy

The firmware was written in MicroPython for the RP2040. The sampling strategy is designed to capture clean RMS values of the AC waveform:

Sampling Rate: 5 kHz

Samples per Measurement Window: 500 samples

Window Duration: 20 ms (one full 50 Hz cycle)

Samples per Half Cycle: ~10 samples (sufficient for basic RMS)

Note: Increasing the sampling frequency and sample count would allow harmonic analysis and power quality measurement in future revisions.

## RMS Calculation Algorithm

The DC offset removal and RMS calculation pipeline:

1. Collect raw ADC samples

2. Sum all samples → calculate DC offset (average)

3. Subtract DC offset from each sample → true AC waveform

4. Square each sample

5. Average the squared values → mean square

6. Take square root → RMS value

## Configuration (power_meter.py)

## Configuration

SAMPLES = 500

SAMPLE_DELAY_US = 200

ADC_VREF = 3.3

ADC_COUNTS = 65535

## Calibration

VCAL = 268.97

ICAL = 16.667 / 3

## Hardware Init

adc_v = ADC(Pin(26)) # Voltage input

adc_i = ADC(Pin(27)) # Current input

# 6. Known Issues & Limitations

## Hardware Issues

- The RP2040 QFN package was hand-soldered without solder paste or stencil. A few pads are likely bridged or lifted, causing incorrect readings on the voltage ADC input (Pin 26). Current measurement (Pin 27) is reading correctly.

- No test pads were included in the PCB design, making it difficult to probe signals for debugging without risking further damage.

- Web logging (originally planned via an ESP8266 module) was dropped due to PCB space and complexity constraints.

## Logistics

- PCB was ordered via an Indian broker to avoid customs delays experienced with direct international shipping. This reduced cost but significantly increased lead time.

- Indian PCB manufacturers were considered but found to be lower quality compared to JLCPCB for this type of fine-pitch board.

## Planned Fixes for Next Revision

- Add test pads for all critical signal nodes (ADC inputs, bias voltage, power rails).

- Use solder paste and a stencil for RP2040 assembly, or specify PCBA service.

- Re-integrate web logging module on the PCB.

- Increase sampling rate for harmonic analysis capability.

# 7. Cost & Timeline

| Item | Details |
| --- | --- |
| Design Phase | ~1 week |
| PCB Procurement & Shipping | 2–3 weeks |
| Soldering & Testing | 2–3 days |
| Total Build Time | ~1 month |
| Total Component Cost | ₹2,500–₹3,000 (excl. labour, electricity, soldering) |

# 8. Team Contributions

| Team Member | Roll No. | Contributions |
| --- | --- | --- |
| Dwaipayan Shikari | 002210801231 | PCB Design, Logistics & Soldering |
| Shramana Mondal | 002210801239 | Firmware Development |
| Prattay Barua | 002210801246 | Logistics, Component Sourcing & Testing |
| Jayrup Das | 002210801131 | Soldering, Testing & 3D Models |

# 9. Repository & References

GitHub Repository: github.com/dwan6767/uniopme

All technical details including full schematics, firmware source code, PCB Gerber files, and 3D models are available in the project repository.

Thank You